Display Panel Indicators

The display panel is located on the front of the PC Scan. There are six indicators that light in different combinations to get the status of the PC Scan.

When you first turn the PC Scan on, all of the indicators and the number 8 are illuminated during the self-test. When the PC Scan is turned off, the indicators are not visible.

[image: image13.png]



The Power-On, Wait and Ready indicators are located in the lower-left portion of the display panel and indicate whether or not the PC Scan is ready to read pages.

The Code indicator in the center of the panel provides you with specific information on functions the PC Scan is performing or what kind of operator assistance or service is required.

The Operator Assistance Required and Service Required lights indicate that the PC Scan requires attention before it can continue to operate.

The color of the indicator lights is also functional: green for Power-On and Ready, yellow for Wait, and red for indicators displayed when the PC Scan requires attention.

Normal Operation Indicators

The following are illustrations of indicators displayed during normal operation.

Status: Self-Test

[image: image1.png]Operator
——— Assistant
Required

Senice
Required

Wait

Pawer-On



The 8 code and all of the indicators are illuminated when the power switch of the PC Scan is turned on. The indicators remain on while the PC Scan performs a self-test to make sure all of its functions are operating properly

Status: Waiting to load

[image: image2.wmf] 

When your PC Scan has completed its self-test, a 0 code is illuminated along with the indicators shown in this illustration. This display indicates that the PC Scan is waiting for software to be loaded.

Status: Ready to Read Pages

[image: image3.png]


After your PC Scan software program is loaded (see software user’s guide), the two green lights are illuminated as shown in the illustration. These lights indicate that the PC Scan is ready to read pages. While a page is being read, the green Ready light is replaced temporarily by the yellow Wait light.

Operator Assistance Required Conditions

When the PC Scan cannot read pages, the Operator Assistance Required indicator comes on until the condition is corrected. A code number that identifies the condition is also displayed.

The following are example of Operator Assistance Required Conditions that may be observed on the display panel of the PC Scan.

Status: Misfeed

[image: image4.png]


Explanation: A page is jammed inside the PC Scan, or the page inserted was longer that 14 inches.

Action: Refer to your PC Scan software user’s guide for recommended action.

Status: Hard-To-Read Page
[image: image5.wmf] 

Explanation: The text page being read contains nontyped material or a typestyle not easily recognized by the PC Scan. The reading rate for the page is very slow.

Action: The display panel will clear after the page is read. Refer to you PC Scan software user’s guide for procedure to cancel page and other helpful hints.

Status: Communications Interrupted


[image: image6.png]


Explanation: Communication with the PC Scan was disrupted while a page was being read.

Action: Refer to your PC Scan software user’s guide for recommended action.

Status: Invalid Switches

[image: image7.png]


Explanation: The switches on the back of the PC Scan are set incorrectly.

Action: Turn the PC Scan off. Set all of the switches on the back of the PC Scan to the correct setting as shown in the installation section of this guide. Turn the PC Scan on and load PC Scan software.
 

Status: Memory Error

[image: image8.png]


Explanation: A memory error has been detected on the PC Scan.

Action: Turn the PC Scan off and back on. If it completes the self-test, load PC Scan software. If the error condition persists, contact your authorized DEST dealer for service.

Status: Data Error

[image: image9.png]


Explanation: A data error was detected while software was being loaded into the PC Scan’s memory.

Action: Turn the PC Scan off and back on. If it completes the self-test, load PC Scan software. If it does not complete self-test, reinstall PC Scan software on your computer, then turn on the PC Scan. If it completes self-test, load PC Scan software. If the error condition persists, contact your authorized DEST dealer for service.

Service Required Conditions

The Service Required indicator is on when the PC Scan detects a problem that requires service. This type of problem is detected by the PC Scan’s self-test diagnostics when the PC Scan is first turned on or while it is waiting to be used.

The following are examples of Service Required Conditions that may be observed:

Status: Scanner Controller Parity Error

[image: image10.png]


Explanation: Text cannot be transmitted due to a parity error.

Action: Refer to your PC Scan software user’s guide for recommended action.

Status: Service Required

[image: image11.png]


When a code other than C is displayed, note the code letter and turn the PC Scan off. Contact your authorized DEST dealer for service.

� EMBED Word.Picture.8  ���








[image: image12.png]


_1042321223.doc
[image: image1.png]






